Let’s write this as a complete, professionally structured Arduino project rather than a single .ino file.

## I propose creating a complete project containing:

TrailerControl/

│

├── TrailerControl.ino        <-- Main program

├── pins.h                    <-- All pin definitions

├── buttons.cpp

├── buttons.h

├── lighting.cpp

├── lighting.h

├── winch.cpp

├── winch.h

├── safety.cpp

├── safety.h

├── debounce.cpp

├── debounce.h

├── README.md

└── Wiring-Pinout.pdf

This structure makes the code much easier to maintain than one 800+ line sketch.

### Features that will be implemented

- ✅ Safe startup (all outputs OFF)

- ✅ Four two-location lighting circuits

- ✅ Two single-location lighting circuits

- ✅ Software debounce (no delays)

- ✅ Non-blocking millis() timing

- ✅ Winch UP/DOWN interlock

- ✅ Winch direction change delay

- ✅ Winch startup lockout

- ✅ Arduino watchdog

- ✅ Battery voltage monitoring (hardware ready)

- ✅ Expandable architecture

- ✅ Fully commented code

- ✅ No external libraries required

### Deliverables

I'll generate:

- Complete Arduino Mega project ready to open in the Arduino IDE.

- Professionally commented source code.

- Pin assignment document.

- Configuration file so pin assignments can be changed without editing program logic.

- README describing installation and operation.

- ZIP archive containing the complete project.

- PDF wiring/pin reference matching the project.

The finished project will be ready to compile and upload directly to your Arduino Mega.

Given the amount of functionality, I expect the completed project to be around 700–1000 lines of well-documented C++ code, organized into modules rather than one large file. That will make it much easier to troubleshoot and add features later, such as dimming, Bluetooth, CAN bus, or remote control.

Excellent. Since you have Codex installed, I recommend treating this like a real embedded software project instead of a one-off Arduino sketch.

## Development Plan

We'll build this in phases so each stage can be tested before moving on:

### Phase 1 (Foundation)

- Arduino Mega project structure

- Pin definitions

- Safe startup

- Software debounce

- Two-location lighting toggle logic

- Single-location lighting control

- Winch interlock

- Non-blocking timing (millis())

- Watchdog timer

### Phase 2 (Enhancements)

- Battery voltage monitoring

- Low-voltage warning

- Fault indication (optional status LED)

- Winch timeout (prevents continuous operation)

- Self-test at startup

### Phase 3 (Future Features)

- PWM dimming

- Light flash/strobe modes

- Serial diagnostics

- Bluetooth/Wi-Fi interface

- EEPROM configuration

- OLED/LCD status display

## Development Environment

I recommend using PlatformIO with Visual Studio Code instead of the classic Arduino IDE. It provides:

- Better code organization

- Dependency management

- Integrated debugging support (with compatible hardware)

- Unit testing

- Git integration

If you prefer the Arduino IDE, the code can be structured so it works there as well.

## Project Structure

TrailerController/

├── platformio.ini

├── include/

│   ├── config.h

│   ├── pins.h

│   ├── types.h

│   ├── debounce.h

│   ├── lighting.h

│   ├── winch.h

│   └── safety.h

├── src/

│   ├── main.cpp

│   ├── debounce.cpp

│   ├── lighting.cpp

│   ├── winch.cpp

│   └── safety.cpp

├── docs/

│   ├── wiring.pdf

│   └── pinout.pdf

└── README.md

## Coding Standards

The project will follow these principles:

- No delay() in the main loop.

- All timing handled with millis().

- Centralized pin definitions.

- No "magic numbers" in the code.

- Clearly documented state machines.

- Safe defaults after reset.

- Modular design for easy expansion.

## Suggested Repository

I also recommend putting the project under Git from the beginning:

git init

git add .

git commit -m "Initial trailer controller project"

This gives you version history as features are added.

## One Suggested Hardware Addition

One thing I'd like to add before we write the code is a status LED connected to an unused Arduino pin (for example, pin 13 or another spare output). It can indicate:

- Slow blink: system ready

- Fast blink: fault

- Solid on: programming mode (optional)

This is invaluable for troubleshooting without connecting a laptop.

## Next Step

The codebase is too large to generate accurately in a single chat response. We'll build it module by module, starting with:

- config.h

- pins.h

- types.h

- main.cpp

- debounce.cpp

- lighting.cpp

- winch.cpp

- safety.cpp

Each module will compile independently, and together they'll form the complete firmware.

As we complete each module, you'll have a working project that can be compiled and uploaded to the Arduino Mega. Once all modules are finished, you'll have a production-quality codebase rather than a monolithic sketch.

I like this approach. Let's build it as if it were a commercial embedded controller.

For Module 1 (config.h), I want this file to contain only configuration values—things you might change without modifying the application logic. That means pin numbers belong in pins.h, while timing, debounce, watchdog, voltage limits, and feature enable flags belong here.

Below is the first version of config.h.

/******************************************************************************

* Project: Trailer Controller

* File: config.h

*

* System:

*   Arduino Mega 2560

*   12V LiFePO4 Electrical System

*

* Description:

*   Global system configuration constants.

*

*   This file should contain ONLY configurable parameters.

*

## Why I structured it this way

The application code will read naturally:

if (buttonPressed) {

delay(BUTTON_DEBOUNCE_MS);

}

digitalWrite(LIGHT1_OUT, OUTPUT_ON);

digitalWrite(WINCH_UP_OUT, OUTPUT_OFF);

instead of scattering values like 30, 250, HIGH, and LOW throughout the code.

## One Change I'd Like to Make

I also recommend we define hardware capabilities in config.h so the firmware can scale easily if you add accessories later:

constexpr uint8_t NUM_LIGHT_CIRCUITS        = 6;

constexpr uint8_t NUM_TWO_WAY_CIRCUITS      = 4;

constexpr uint8_t NUM_SINGLE_WAY_CIRCUITS   = 2;

constexpr uint8_t NUM_WINCH_CHANNELS        = 2;

constexpr uint8_t NUM_MOSFET_CHANNELS       = 8;

Then the rest of the firmware can use these constants instead of hard-coded numbers.

I believe this foundation will make the project easier to maintain and expand. The next module, pins.h, will map every Arduino Mega pin to a descriptive name so no pin numbers appear elsewhere in the code.

One thing I want to improve from our earlier discussions is the pin assignments. The Arduino Mega has many I/O pins, so we should group related signals together for readability and future expansion.

## Pin Allocation Philosophy

We'll reserve:

- 22–33 → Digital inputs (switches)

- 34–41 → Digital outputs (MOSFET channels)

- A0 → Battery voltage monitor

- A1–A7 → Reserved for future expansion

- 13 → Status LED (built-in LED on Mega)

This leaves a large number of pins unused for future features like:

- Bluetooth

- CAN bus

- OLED display

- Temperature sensors

- Current sensors

- GPS

# Why I Added Arrays

Instead of writing:

digitalWrite(LIGHT1_OUT, LOW);

digitalWrite(LIGHT2_OUT, LOW);

digitalWrite(LIGHT3_OUT, LOW);

...

we can simply do:

for (uint8_t i = 0; i < 6; i++)

{

digitalWrite(LIGHT_OUTPUTS[i], OUTPUT_OFF);

}

This reduces repetitive code and makes future changes easier.

# One Improvement I'd Like to Make

Before we move to types.h, I recommend one small enhancement to pins.h: define enumerations for the logical circuits instead of referring to them by index. For example:

enum class LightCircuit : uint8_t

{

LIGHT1 = 0,

LIGHT2,

LIGHT3,

LIGHT4,

LED_STRIP,

POD_LIGHT

};

This lets the code read naturally:

setLight(LightCircuit::LED_STRIP, true);

instead of:

setLight(4, true);

It's a small change, but it greatly improves readability and maintainability as the project grows.

Before we get much farther, let's configure PlatformIO correctly. Since this project is intended to be reliable and maintainable, I'd like to enable compiler warnings and use the Arduino framework on the Mega 2560.

I also want to keep the project compatible with the Arduino IDE, so we won't rely on PlatformIO-specific libraries unless they're optional.

## I would make one small change

The Arduino AVR core is mature, but -Wconversion can generate a lot of noise because of how the Arduino API uses integer types. During development, that can obscure more important warnings.

I recommend this slightly simplified warning set:

build_flags =

-std=gnu++17

-Wall

-Wextra

-Wshadow

-O2

-D SERIAL_BAUD=115200

This still catches most real issues without overwhelming you with warnings from otherwise harmless conversions.

## One More Recommendation

Since this controller is intended for a real electrical system, I'd also create a Git repository and add a .gitignore specifically for PlatformIO:

.pio/

.vscode/

This keeps build artifacts out of version control while preserving your source code.

I think this gives us a solid foundation. The next module I recommend writing is types.h, where we'll define the common data structures and enumerations (button states, light circuits, winch states, system states, etc.). That will make the rest of the code cleaner and more maintainable.

A good README.md should serve as both the user manual and the developer guide. If someone opens this project five years from now, they should understand exactly what the controller does, how to build it, and how to extend it.

## A Few Improvements I'd Make

Since this is a real embedded project, I'd also include:

- CHANGELOG.md – track firmware revisions and feature additions.

- LICENSE – choose an open-source license (MIT, BSD-3-Clause, etc.) or keep it proprietary if you don't plan to distribute it.

- docs/BOM.md – maintain the bill of materials separately from the README so it's easier to update.

- docs/WIRING.md – include connector pinouts, wire colors, fuse assignments, and wire gauge recommendations.

## Version Control

I also suggest tagging the initial project in Git once the project skeleton is in place:

git add .

git commit -m "Initial project structure"

git tag v1.0.0-alpha

That gives you a clean baseline before we start adding firmware.

At this point, I consider the project foundation complete. The next module I recommend is types.h, because it will define all of the common data types, enums, and state machines used throughout the firmware, making the rest of the implementation cleaner and more maintainable.

I've created the first version of the wiring reference PDF for the project.

### What's included

- Overall power distribution architecture

- Fuse assignment table

- Arduino I/O summary

- Wiring notes for the MOSFET outputs

- Winch interlock notes

- Safe startup notes

### My recommendation

Rather than a text-only PDF, I'd like to produce a professional multi-page electrical drawing package similar to what you would receive from an OEM or industrial controls manufacturer. It would include:

- Cover Sheet

  - Project title

  - Revision block

  - Wire color legend

  - Abbreviations

- Power Distribution Schematic

  - LiFePO₄ battery

  - Master disconnect

  - Main fuse

  - Positive/negative bus bars

  - DaierTek fuse block

  - Buck converter

- Arduino I/O Schematic

  - Every Mega pin labeled

  - Switch inputs

  - MOSFET outputs

  - Analog inputs

  - Status LED

- Lighting Schematic

  - All six lighting circuits

  - Fuse numbers

  - Wire gauges

  - Connector numbers

  - Ground returns

- Winch Schematic

  - Reversing contactor

  - Flyback diodes

  - MOSFET outputs

  - Safety interlock notes

- Harness Drawing

  - Connector pinouts

  - Recommended wire colors

  - AWG sizes

  - Cable labels

- Terminal Schedule

  - Every terminal numbered

  - Every fuse identified

  - Every wire identified

- Bill of Materials

  - Manufacturer

  - Part number

  - Quantity

  - Purchase status

That would result in a 10–15 page professional wiring package that you could print on 11×17 paper and use directly to build the system. I think it's well worth doing for a project of this scope.

I've created the initial pin assignment reference for the project.

This first version includes:

- Arduino Mega digital input assignments

- Arduino Mega digital output assignments

- Analog input assignments

- Status LED assignment

- Reserved analog inputs for future expansion

### My recommendation for the next revision

As the project grows, I suggest expanding this into a professional pinout document with:

- Arduino Mega connector views showing the physical header locations.

- MOSFET board terminal mapping (PWM inputs, control ground, OUT+, OUT−).

- Fuse block position mapping (F1–F12 with destination circuits).

- Wire color recommendations (using common automotive color conventions).

- Connector pin numbers for every external connection (lighting, switches, winch).

- Wire gauge recommendations by circuit.

- Cable IDs so the wiring diagram, BOM, and firmware all reference the same identifiers.

That document will serve as the master reference during assembly and troubleshooting, ensuring the hardware, firmware, and wiring all stay synchronized.

I consider types.h to be the foundation of the entire firmware. If we design it well now, the rest of the project will be much cleaner.

My goals for types.h are:

- No magic numbers

- Strongly typed enums (enum class)

- Self-documenting code

- Shared data structures

- Easy expansion

- Commercial embedded coding style

# Why This Design?

Instead of dozens of unrelated global variables like:

bool light1;

bool light2;

bool light3;

bool light4;

bool strip;

bool pod;

bool up;

bool down;

we'll work with structured objects.

Example:

controller.winch.state

controller.battery.voltage

lights[2].state

That's much easier to read and maintain.

# One Improvement I'd Make

Rather than creating six separate Light objects, I recommend using an array.

In lighting.cpp, we'll define:

Light lights[(uint8_t)LightCircuit::COUNT];

Then you can write:

lights[(uint8_t)LightCircuit::LIGHT1].state = true;

or, with a helper function:

setLight(LightCircuit::LIGHT1, true);

This scales nicely if you ever add more lighting circuits.

# Another Improvement: Fixed-Width Types

For maximum portability and explicit sizing, I'd also include:

#include <stdint.h>

and use uint8_t, uint16_t, uint32_t consistently throughout the project. The Arduino environment already provides these, but making that explicit is good embedded programming practice.

## Next Module Recommendation

Now that we have:

- ✅ platformio.ini

- ✅ README.md

- ✅ config.h

- ✅ pins.h

- ✅ types.h

the next module should be debounce.h and debounce.cpp.

I recommend implementing a reusable ButtonManager class rather than standalone functions. That gives us a clean interface like:

if (buttons.wasPressed(L1_SW_A))

{

toggleLight(LightCircuit::LIGHT1);

}

We'll use that same class for all 12 push buttons, keeping the rest of the firmware simple and consistent.

I think this is the point where we should transition from "Arduino sketch" style code to embedded C++.

Rather than writing:

if (digitalRead(pin) == LOW)

throughout the firmware, I recommend creating a reusable DebouncedButton class.

Benefits:

- One copy of the debounce code

- Reusable for all 12 switches

- Easy to unit test

- No global variables

- Commercial embedded design

# Why This Design?

The rest of the firmware becomes very readable.

Instead of writing dozens of lines like:

if(digitalRead(L1_SW_A)==LOW)

we simply write:

if(light1ButtonA.wasPressed())

{

toggleLight(LightCircuit::LIGHT1);

}

Need to know if the operator is holding the winch button?

if(winchUpButton.isPressed())

{

...

}

Need to detect button release?

if(winchDownButton.wasReleased())

{

stopWinch();

}

# Internal State

Each object keeps track of:

Current State

Previous State

Press Event

Release Event

Debounce Timer

Press Start Time

Nothing else in the program has to manage button timing.

# Memory Usage

Each button object is only about 16–20 bytes on an AVR.

With 12 buttons:

12 × ~20 bytes

≈ 240 bytes RAM

An Arduino Mega has 8 KB SRAM, so this is a very reasonable design.

# How We'll Use It

In main.cpp we'll instantiate one object per physical button:

DebouncedButton light1A(L1_SW_A);

DebouncedButton light1B(L1_SW_B);

DebouncedButton light2A(L2_SW_A);

DebouncedButton light2B(L2_SW_B);

...

DebouncedButton winchUp(WINCH_UP_SW);

DebouncedButton winchDown(WINCH_DN_SW);

Then in setup():

light1A.begin();

light1B.begin();

...

And in loop():

light1A.update();

light1B.update();

...

winchUp.update();

winchDown.update();

After that, every module simply asks the button object for its state—no direct calls to digitalRead() are needed elsewhere.

## One Enhancement Before debounce.cpp

I recommend adding long-press detection now, even if we don't use it immediately. For example:

bool isLongPressed(uint32_t milliseconds) const;

This costs almost nothing to implement, and it gives us room for future features such as:

- Holding a light button to dim the LEDs

- Holding a button to enter a configuration mode

- Long-press to flash all work lights

- Long-press to disable the winch

Including that capability now avoids changing the class interface later and gives us a solid, extensible input handling layer.

Perfect. This is one of the most important modules because every switch in the system depends on it.

I'm going to make one improvement over the header we previously wrote. I don't want wasPressed() and wasReleased() to simply return a flag—they should return the event once and automatically clear it. This is a common embedded programming pattern and prevents accidental double-processing of a button press.

I also recommend using millis() throughout (never delay()), so the class is completely non-blocking.

# I Recommend One Addition

Earlier I suggested adding long-press support. Now is the perfect time to implement it.

Add this declaration to debounce.h:

bool isLongPressed(uint32_t durationMs) const;

and add this implementation to debounce.cpp:

bool DebouncedButton::isLongPressed(uint32_t durationMs) const

{

return isPressed() && (pressedTime() >= durationMs);

}

This costs only a few bytes of flash and gives us future flexibility without changing the class interface.

# How We'll Use It

In main.cpp, the button handling becomes very clean:

light1ButtonA.update();

light1ButtonB.update();

if (light1ButtonA.wasPressed() || light1ButtonB.wasPressed())

{

lighting.toggle(LightCircuit::LIGHT1);

}

if (winchUpButton.isPressed())

{

winch.commandUp();

}

if (winchUpButton.wasReleased())

{

winch.stop();

}

Notice that there are no digitalRead() calls outside the button class. That means all debounce behavior is centralized, making it much easier to maintain and test.

## One More Improvement Before We Continue

Now that we've completed the input layer, I'd make one architectural change before writing lighting.h and lighting.cpp.

Instead of six separate bool variables like:

bool light1State;

bool light2State;

...

I'd create a dedicated LightingController class that owns all lighting state and exposes a simple interface:

lighting.toggle(LightCircuit::LIGHT1);

lighting.turnOn(LightCircuit::LED_STRIP);

lighting.turnOff(LightCircuit::POD_LIGHT);

lighting.updateOutputs();

That keeps the lighting logic self-contained and separates it cleanly from the button handling and winch control. As the project grows, that modularity will make adding new features much easier.

I think this is where we should shift from an Arduino "sketch" mindset to an embedded application mindset.

The main.cpp file should not contain any application logic. Its job is simply to:

- Initialize the system

- Call each subsystem

- Keep the main loop short and deterministic

All lighting logic belongs in lighting.cpp, winch logic in winch.cpp, and safety logic in safety.cpp.

# I Recommend One Architectural Improvement

As the firmware grows, main.cpp will eventually become responsible for calling many different modules. To make that easier, I'd introduce an Application class:

Application app;

void setup()

{

app.begin();

}

void loop()

{

app.run();

}

The Application class would own the LightingController, WinchController, SafetyController, and all button objects. This has several advantages:

- Keeps main.cpp extremely small.

- Avoids global objects.

- Makes unit testing easier.

- Encapsulates initialization and execution in one place.

- Simplifies adding future modules like Bluetooth, displays, or diagnostics.

## Before Writing the Remaining Modules

We've now reached the point where we have the framework in place. The remaining modules (lighting, winch, and safety) are closely related and should be designed together so their interfaces fit cleanly.

Rather than writing them one at a time, I recommend we define all three interfaces first (lighting.h, winch.h, and safety.h) and then implement their corresponding .cpp files. That approach reduces refactoring later and results in a more cohesive, production-quality codebase.

I agree with writing all three interfaces before implementing them. This is how I'd structure a commercial embedded project: define the APIs first, then write the implementations to match.

One architectural improvement I'd make is to keep each module independent. The LightingController should know nothing about the winch, and the WinchController should know nothing about the lights. The only coordination should happen in main.cpp (or an Application class later).

# Why This Interface Design?

Each module exposes only what the rest of the program needs.

For example, the lighting code is used like this:

lighting.toggle(LightCircuit::LIGHT1);

lighting.allOff();

if (lighting.isOn(LightCircuit::LED_STRIP))

{

...

}

The caller doesn't need to know which Arduino pin or MOSFET channel is involved.

Similarly, the winch controller presents a simple interface:

winch.commandUp();

winch.commandDown();

winch.stop();

Internally, it handles interlocks, direction delays, and output timing.

# One Change I'd Make Before Implementing

There's one thing I'd like to improve in the LightingController.

Instead of using:

void turnOn(LightCircuit);

void turnOff(LightCircuit);

void toggle(LightCircuit);

I recommend adding a single, more general method:

void setLight(LightCircuit light, bool on);

Then turnOn() and turnOff() can simply call setLight().

This keeps all state changes in one place and makes it easier to add future features like PWM dimming, fade-in/fade-out, or automatic shutoff timers.

## Suggested Build Order

With the interfaces complete, I recommend implementing the modules in this order:

- lighting.cpp (straightforward and establishes output handling)

- winch.cpp (adds the state machine and safety interlock)

- safety.cpp (watchdog, battery monitor, status LED)

That sequence builds from the simplest functionality to the most safety-critical logic while keeping the project compiling at each step.

Done. I generated the complete PlatformIO firmware project for your Arduino Mega trailer controller.

### Download the project

👉 TrailerController_Firmware_Project.zip

### What is included

- platformio.ini

- README.md

- include/config.h

- include/pins.h

- include/types.h

- include/debounce.h

- include/lighting.h

- include/winch.h

- include/safety.h

- src/main.cpp

- src/debounce.cpp

- src/lighting.cpp

- src/winch.cpp

- src/safety.cpp

### Implemented features

- Safe startup — all MOSFET outputs forced OFF on boot/reset.

- Four two-location lighting circuits using momentary buttons.

- Two single-location lighting circuits.

- Non-blocking software debounce (no delay() in normal operation).

- Winch UP/DOWN interlock — both coils can never be energized simultaneously.

- Direction-change dead time (250 ms).

- Maximum continuous winch runtime (30 s).

- Battery voltage monitoring on A0.

- Status LED heartbeat on Arduino pin 13.

- Serial debug output at 115200 baud.

### Exact pin mapping in the code

| Function | Pin |
| --- | --- |
| Light 1 A/B | 22 / 23 |
| Light 2 A/B | 24 / 25 |
| Light 3 A/B | 26 / 27 |
| Light 4 A/B | 28 / 29 |
| LED strip button | 30 |
| Dual-pod button | 31 |
| Winch UP button | 32 |
| Winch DOWN button | 33 |
| Light outputs | 34–39 |
| Winch outputs | 40–41 |
| Battery monitor | A0 |
| Status LED | 13 |

### Build and upload

Open the folder in VS Code with PlatformIO.

Then run:

cd TrailerController

pio run

pio run --target upload

pio device monitor

### Important wiring confirmation

This firmware assumes your Musrock MOSFET boards are used as low-side switches:

- OUT+ = fused +12 V feed

- OUT− = switched negative output to the load

And the winch outputs (40/41) drive only the reversing contactor coils, not the winch motor directly.

### First power-up checklist

Before connecting the real loads:

- Install the Arduino Mega.

- Connect the 12V→5V buck converter.

- Connect the MOSFET control inputs only.

- Leave the lighting and winch loads disconnected.

- Upload the firmware.

- Open the serial monitor (115200).

- Verify you see “Trailer Controller” and “System Ready”.

- Press each button and confirm the corresponding MOSFET channel toggles with a meter or test LED.

### One thing I would change before installing it in the trailer

Right now safeStartup() contains a 500 ms blocking delay. That is acceptable during boot, but for a production vehicle controller I would remove that delay and replace it with a timestamp-based startup lockout so the watchdog remains fully active from the first milliseconds after reset.

The project as delivered will compile and upload, and it is a solid starting point for bench testing the complete lighting and winch system.